LKA System — Architecture Document

Lane Keeping Assist | CarMaker 15 + MATLAB/Simulink R2025a

Version 1.0   |   May 2026

# 1. What Is the System Architecture Document?

The System Architecture Document (SAD) describes the structure, components, data flows, and design decisions of the Lane Keeping Assist (LKA) system. It is NOT the LKA_System.slx Simulink model — the .slx file is the implementation. This document explains:

- What each subsystem does and why it exists

- How data flows between CarMaker, Simulink, and the vehicle actuators

- The algorithms and mathematical models used at each stage

- Design decisions and trade-offs made during development

- The relationship between software blocks and physical sensors/actuators

# 2. System Overview

The LKA system keeps a vehicle centered in its lane by processing camera images and computing steering angles in real time. It runs inside a MATLAB/Simulink model (LKA_System.slx) embedded in CarMaker's generic.mdl wrapper.

## 2.1 Top-Level Data Flow

| Stage | Component | Input | Output |
| --- | --- | --- | --- |
| 1 | CarMaker Sensor | 3D simulation | RGB image 480x640x3 |
| 2 | IPGMovie_Capture | CarMaker camera | uint8 image frame |
| 3 | CarMaker_Input | Camera + vehicle state | lane_params bus |
| 4 | Perception | camera_img [480x640x3] | lane_params struct |
| 5 | Error_Computation | lane_params + vehicle_state | error_signals struct |
| 6 | Adaptive_Lookahead | speed + error_signals | Ld [scalar, meters] |
| 7 | Pure_Pursuit | lane_params + Ld | delta [rad] |
| 8 | Steering_Filter | raw delta | filtered delta |
| 9 | Anti_Ackermann | filtered delta | FL, FR angles [rad] |
| 10 | CarMaker_Output | steering_cmd [5x1] | VhclCtrl.Steering |

# 3. Subsystem Descriptions

## 3.1 CarMaker_Input Subsystem

Receives raw data from the CarMaker simulation environment via two placeholder blocks:

- Camera_Placeholder: Receives uint8 RGB image [480x640x3] from IPGMovie_Capture block

- State_Placeholder: Receives vehicle state bus (speed, X, Y, Yaw, Roll, Pitch, YawRate, az)

NOTE: These placeholders are wired in generic.mdl, not in LKA_System.slx directly.

## 3.2 Perception Subsystem

Core vision processing pipeline that converts a raw RGB camera image into lane polynomial fits.

- ROI Crop: Retains rows 45%-90% of image height to remove sky and close-to-vehicle regions

- BEV Transform: Projective transform to bird's-eye view using fitgeotrans (src/dst trapezoid)

- HSV Segmentation: Detects white (S<0.18, V>0.75) and yellow (H=0.05-0.18) lane markings

- Sliding Window: 9 windows, polynomial-guided search using prev_fit prediction at each height

- Polyfit: 2nd-order polynomial x = ay^2 + by + c fitted separately for left and right lanes

- Single-line fallback: If one lane lost, extrapolates using stored lane width

- Temporal smoothing: EMA filter alpha=0.20 applied to polynomial coefficients

## 3.3 Error_Computation Subsystem

Computes three error signals from the center polynomial evaluated at the vehicle position:

- lateral_error = (vehicle_x - polyval(center_fit, bev_height)) * xm_per_pix [meters]

- heading_error = atan(2*a*bev_height + b) [radians]

- curvature = 2*a [1/pixels]

## 3.4 Adaptive_Lookahead Subsystem

Computes a speed-adaptive lookahead distance Ld that reduces on curves and with high lateral error:

- Ld = Ld_base + k_speed * speed

- Ld = Ld / (1 + k_curv * curvature)  [k_curv=800 for pixel curvature units]

- Ld = Ld - k_err * |lateral_error|

- Clamped to [0.8, 4.0] meters

## 3.5 Pure_Pursuit Subsystem

Implements the geometric Pure Pursuit controller with hybrid PID correction:

- Evaluates center_fit at goal_y = bev_height - Ld/ym_per_pix

- alpha = atan2(lateral_offset_m, Ld)

- delta_pp = atan(2 * wheelbase * sin(alpha) / Ld)

- Heading feed-forward: heading_ff = atan(lane_slope) * 0.25

- PID on pixel error with gain scheduling (straight vs curved road)

- Final: delta = delta_pp + delta_pid + heading_ff, clamped to ±0.436 rad (±25°)

## 3.6 Steering_Filter Subsystem

Low-pass filter with rate limiter to eliminate high-frequency oscillations in the steering command.

## 3.7 Anti_Ackermann Subsystem

Converts a single steering angle to individual front-left and front-right wheel angles using BAJA buggy geometry:

- R = wheelbase / tan(|delta|)

- R_inner = R - track_width/2,  R_outer = R + track_width/2

- delta_inner = atan(wheelbase / R_inner),  delta_outer = atan(wheelbase / R_outer)

- Anti-Ackermann: outer wheel angle > inner wheel angle (opposite to standard cars)

- Left turn: FL = delta_outer, FR = delta_inner

## 3.8 CarMaker_Output Subsystem

Packages the steering command as a 5-element vector for CarMaker's vehicle control interface:

- steering_cmd = [delta_FL; delta_FR; 0; 0; 0]

- Maps to VhclCtrl.Steering in generic.mdl

# 4. Key System Parameters

| Parameter | Value | Location | Description |
| --- | --- | --- | --- |
| wheelbase | 2.0 m | Pure_Pursuit | BAJA buggy axle separation |
| track_width | 1.6 m | Anti_Ackermann | BAJA buggy wheel separation |
| max_steer | 0.436 rad (25°) | Pure_Pursuit | Physical steering limit |
| xm_per_pix | 0.0083 m/px | Perception/Error | Lateral scale in BEV |
| ym_per_pix | 0.010 m/px | Pure_Pursuit | Longitudinal scale in BEV |
| Ld_base | 1.8 m | Adaptive_Lookahead | Base lookahead distance |
| k_curv | 800 | Adaptive_Lookahead | Curvature gain (pixel units) |
| alpha | 0.20 | Perception | EMA smoothing factor |
| nwindows | 9 | Perception | Sliding window count |
| margin | 100 px | Perception | Sliding window half-width |
| vehicle_center | 320 px | Pure_Pursuit | Image center x-coordinate |
| bev_height | 240 px | Pure_Pursuit | BEV image height |