LKA System — Sensor Configuration File

CarMaker 15.0   |   Vehicle: BAJA_Honda_LKA   |   May 2026

# 1. Camera Sensor — FrontCamera (CA00)

Primary sensor for lane detection. Provides RGB images to the Perception subsystem.

## 1.1 Physical Mounting

| Parameter | Value | Notes |
| --- | --- | --- |
| Sensor Name | FrontCamera / CA00 | CarMaker sensor ID |
| Body Mounting | Frame Fr1A | Vehicle body reference frame |
| Position X | 0.5 m | Forward from vehicle origin |
| Position Y | 0.0 m | Centerline (no lateral offset) |
| Position Z | 1.2 m | Height above ground |
| Roll | 0.0 deg | No side tilt |
| Pitch | 15.0 deg (down) | Camera tilted toward road |
| Yaw | 0.0 deg | Facing forward |

## 1.2 Image Properties

| Parameter | Value | Notes |
| --- | --- | --- |
| Image Width | 640 pixels | Horizontal resolution |
| Image Height | 480 pixels | Vertical resolution |
| Color Format | RGB24 | 3-channel uint8, 0-255 |
| Frame Rate | 30 Hz | One image every 33 ms |
| Cycle Time | 33 ms | Set in CarMaker Sensors tab |
| Field of View | 60 deg (approx) | Horizontal FOV |
| Active | Yes (checked) | Must be enabled in CarMaker |

## 1.3 BEV Calibration (Perception Subsystem)

The following source/destination point pairs define the projective transform used in the Perception function. These must be calibrated to the actual camera view of the road.

| Point | Source (Camera) | Destination (BEV) | Description |
| --- | --- | --- | --- |
| Top-Left | [220, 20] | [160, 0] | Near-horizon left edge |
| Top-Right | [420, 20] | [480, 0] | Near-horizon right edge |
| Bottom-Right | [540, 210] | [480, 240] | Near-vehicle right edge |
| Bottom-Left | [100, 210] | [160, 240] | Near-vehicle left edge |

Pixel-to-Meter Calibration:

| Parameter | Value | Derivation |
| --- | --- | --- |
| xm_per_pix (lateral) | 0.0083 m/px | 2.0 m lane / ~241 px BEV width |
| ym_per_pix (longitudinal) | 0.010 m/px | Calibrated to road geometry |

# 2. Line Sensor — Vhcl.LN00 (VehSensor_1)

Used by CarMaker's Generic Lateral Control (IPG built-in LKA). Must be DISABLED when using Simulink LKA to prevent conflicts.

## 2.1 Configuration

| Parameter | Value | Notes |
| --- | --- | --- |
| Sensor Name | Vhcl.LN00 | EXACT name required (capital V) |
| Sensor Type | Line Sensor | Lane boundary detector |
| Sensor ID | VehSensor_1 | CarMaker internal reference |
| Position X | 2.5 m | At front bumper |
| Position Y | 0.0 m | Centerline |
| Position Z | 0.3 m | Low to ground |
| Detection Range | 50 m | Forward detection distance |
| Cycle Time | 35 ms | ~28 Hz |
| Active | Yes | Must exist for CarMaker |
| Visualization | DISABLED | Overlay contaminates camera image! |

## 2.2 CRITICAL: Visualization Must Be Off

When the LineSensor visualization is enabled, CarMaker renders RED and GREEN overlay lines directly into the IPGMovie camera view. These colored lines are then detected by the Perception function as false lane markings, causing:

- False lane width of ~230 px (should be ~400 px)

- Detected center always at ~320 px regardless of vehicle position

- Zero lateral error computed on every frame

- Zero steering command output — vehicle makes no correction

FIX: In CarMaker Vehicle Editor → Sensors → Vhcl.LN00 → Uncheck Visualize

# 3. Vehicle State Signals (CarMaker → Simulink)

These signals are mapped from CarMaker quantities to the State_Placeholder input of LKA_System in generic.mdl.

| CarMaker Signal | Simulink Field | Units | Description |
| --- | --- | --- | --- |
| Car.vx or Car.v | vehicle_state.speed | m/s | Longitudinal vehicle speed |
| Car.X | vehicle_state.x | m | Global X position |
| Car.Y | vehicle_state.y | m | Global Y position |
| Car.Yaw | vehicle_state.heading | rad | Vehicle heading angle |
| Car.Roll | vehicle_state.roll | rad | Vehicle roll angle |
| Car.Pitch | vehicle_state.pitch | rad | Vehicle pitch angle |
| Car.YawRate | vehicle_state.yaw_rate | rad/s | Yaw rate |
| Car.az | vehicle_state.vert_accel | m/s^2 | Vertical acceleration |

# 4. Steering Output Signals (Simulink → CarMaker)

| steering_cmd Index | CarMaker Actuator | Value Source | Description |
| --- | --- | --- | --- |
| [0] = steering_cmd(1) | VhclCtrl.Steering.Ang.FL | Anti_Ackermann delta_left | Front-left wheel angle [rad] |
| [1] = steering_cmd(2) | VhclCtrl.Steering.Ang.FR | Anti_Ackermann delta_right | Front-right wheel angle [rad] |
| [2] = steering_cmd(3) | VhclCtrl.Steering.Ang.RL | Constant 0 | Rear-left (not steered) |
| [3] = steering_cmd(4) | VhclCtrl.Steering.Ang.RR | Constant 0 | Rear-right (not steered) |
| [4] = steering_cmd(5) | VhclCtrl.Steering.Mode | Constant 0 | Normal steering mode |