LKA System — Hardware-Algorithm Compatibility Report

Version 1.0   |   CarMaker 15.0 + MATLAB R2025a   |   May 2026

# 1. Purpose of This Report

This report documents the compatibility between the physical hardware configuration (sensors, vehicle, road) and the LKA algorithm implementation (Simulink functions). It identifies known incompatibilities, resolved issues, and remaining constraints.

# 2. Platform Compatibility Matrix

| Component | Required Version | Actual Version | Status | Notes |
| --- | --- | --- | --- | --- |
| CarMaker | 12.0+ | 15.0 | COMPATIBLE | Full feature support |
| MATLAB | R2020a+ | R2025a | COMPATIBLE | All toolboxes available |
| Simulink | R2020a+ | R2025a | COMPATIBLE | codegen supported |
| Image Processing TB | Required | Installed | COMPATIBLE | fitgeotrans, imwarp |
| Computer Vision TB | Optional | Check install | VERIFY | bwareaopen fallback exists |
| CarMaker for Simulink | 15.0 | 15.0 | COMPATIBLE | generic.mdl integration |
| Windows OS | Win10/11 | Win11 | COMPATIBLE | App Control policy — run as Admin |

# 3. Sensor-to-Algorithm Compatibility

## 3.1 Camera → Perception Function

| Property | Camera Output | Algorithm Expects | Compatible? | Action Required |
| --- | --- | --- | --- | --- |
| Resolution | 640x480 | 640x480 | YES | None |
| Channels | 3 (RGB24) | 3 channels | YES | None |
| Data type | uint8 | uint8 | YES | None |
| Frame rate | 30 Hz | 30 Hz (0.033s) | YES | Match simulation dt |
| Color space | RGB | Converted to HSV | YES | rgb2hsv inside fcn |
| Lane color (white) | White solid | HSV: S<0.18, V>0.75 | YES | Confirmed working |
| Lane color (yellow) | Yellow double | HSV: H=0.05-0.18 | YES | Confirmed working |
| LineSensor overlay | RED+GREEN lines | Not expected | NO | DISABLE visualization! |
| BEV field of view | ~15m ahead | Needs 10-15m | YES | Verify at speed >30km/h |

## 3.2 Vehicle State → Error Computation

| Signal | CarMaker Unit | Algorithm Unit | Conversion | Compatible? |
| --- | --- | --- | --- | --- |
| Speed (vx) | m/s | m/s | None needed | YES |
| Yaw angle | rad | rad | None needed | YES |
| Roll angle | rad | rad | Input to fcn, not used yet | YES |
| Pitch angle | rad | rad | Input to fcn, not used yet | YES |
| YawRate | rad/s | rad/s | Used in adaptive Ld | YES |
| Vert. accel (az) | m/s^2 | m/s^2 | Used in adaptive Ld | YES |

## 3.3 Steering Output → CarMaker Actuator

| Property | Algorithm Output | CarMaker Expects | Compatible? | Notes |
| --- | --- | --- | --- | --- |
| Format | 5x1 double vector | VhclCtrl.Steering bus | YES (via Mux) | Map in generic.mdl |
| FL angle range | ±0.436 rad | ±0.436 rad max | YES | Clamped in PP fcn |
| FR angle range | ±0.436 rad | ±0.436 rad max | YES | Clamped in PP fcn |
| Rear angles | 0 rad | 0 rad (FWD vehicle) | YES | Not steered |
| Steering mode | 0 | 0 = normal | YES | No special mode |
| Update rate | Every sim step | Every step | YES | Synchronous |
| Anti-Ackermann | FL ≠ FR | Accepts individual | YES | Correct geometry |

# 4. Known Incompatibilities and Resolutions

| Issue ID | Description | Root Cause | Status | Resolution |
| --- | --- | --- | --- | --- |
| IC-01 | LineSensor visualization contaminates camera | RED/GREEN overlay rendered in camera view | RESOLVED | Disable LineSensor visualization in CarMaker |
| IC-02 | green_mask detected CarMaker overlay as lane | H=0.25-0.38 matched both road markings and overlay | RESOLVED | Removed green/orange mask; use white+yellow only |
| IC-03 | center_fit lost curvature on curves | Output was [0;0;centerx] discarding a,b terms | RESOLVED | Now outputs full averaged polynomial |
| IC-04 | Temporal lag 167ms at alpha=0.80 | Too much weight on previous polynomial | RESOLVED | alpha reduced to 0.20 |
| IC-05 | Far-field sliding window missed curved lanes | Fixed-position window, no curve tracking | RESOLVED | Polynomial-guided window search + far-field half-split |
| IC-06 | Width sanity check always failed | eval at y=1 caused polynomial divergence | RESOLVED | Evaluate only at y=Hb/2 to Hb (near-vehicle) |
| IC-07 | Steer generated but not applied to vehicle | IPGDriver lateral control overriding Simulink | RESOLVED | Set Maneuver lateral driver to None |
| IC-08 | Vehicle rollover at simulation start | BAJA vehicle CoG too high | RESOLVED | Using DemoCar/Honda base with BAJA mass |
| IC-09 | LineSensor name case mismatch | vhcl.LN00 vs Vhcl.LN00 | RESOLVED | Use exact name Vhcl.LN00 (capital V) |
| IC-10 | Camera frame frozen in Video Viewer | IPGMovie_Capture sample time mismatch | INVESTIGATE | Set sample time to 0.033s (30Hz) |
| IC-11 | Road orientation mismatch (left vs right) | New scenario uses different lane side | INVESTIGATE | Check laneId sign in xosc, may need error sign flip |

# 5. Road Environment Compatibility

| Road Property | Algorithm Requirement | Current Road | Compatible? | Notes |
| --- | --- | --- | --- | --- |
| Lane marking color | White or Yellow | White (left) + Yellow (right) | YES | Both detected by HSV masks |
| Lane marking width | >5 px in BEV | ~10-15 px in BEV | YES | Visible after bwareaopen |
| Lane width | 2.0 - 5.0 m (240-600px) | ~3.5 m (421 px) | YES | Within sanity check range |
| Road surface color | Dark (contrast) | Dark asphalt | YES | Good contrast for white/yellow |
| Curve radius | >60 m (¦a¦<0.008) | ~100 m typical | YES | Curvature sanity check passes |
| Max curve angle | 20-25 deg | 20-25 deg (xosc scenario) | MARGINAL | Polynomial-guided search helps |
| Road length | Any | 500 m+ (xosc) | YES | No constraint |
| Lighting | Daylight sim | CarMaker default sun | YES | HSV thresholds tuned for this |
| Grass roadside | Must not be detected | Green grass borders | YES | green_mask removed |
| Road speed limit | Up to 30 km/h | 30 km/h test speed | YES | Lookahead Ld adequate |

# 6. Performance Constraints

| Constraint | Limit | Current Performance | Margin |
| --- | --- | --- | --- |
| Max test speed | 30 km/h | 18-30 km/h tested | Within limit |
| Curve handling | 20-25 deg | ~20 deg currently | At limit |
| Lateral error tolerance | <0.5 m | ~0.1-0.3 m (stable) | Within limit |
| Steering smoothness | No jitter | Slight oscillation | Needs filter tuning |
| Lane detection rate | >95% frames | ~95% (both lanes) | ~60% (single lane fallback) |
| Processing latency | <33 ms (1 frame) | ~10-20 ms (MATLAB) | Within limit |
| Algorithm sample time | 0.001 s (1ms) | Set in Simulink | Synchronous with CarMaker |

# 7. Pre-Simulation Compatibility Checklist

Verify all items before running a simulation:

## CarMaker Setup

- [ ] LineSensor (Vhcl.LN00) visualization is DISABLED

- [ ] Camera (CA00) is active with correct position (0.5, 0.0, 1.2) and pitch 15 deg

- [ ] Camera format is RGB24, frame rate 30 Hz

- [ ] Vehicle Control: Generic Lateral Control set to -- not specified --

- [ ] TestRun Maneuver: Lateral driver set to None (not IPGDriver)

- [ ] IPGMovie_Capture sample time: 0.033 s

## Simulink Setup

- [ ] Run setup_lka_workspace() before opening model

- [ ] All bus definitions present in MATLAB workspace

- [ ] Simulation solver: Fixed-step, ode3, dt=0.001 s

- [ ] camera_img port scope: Input (not Output)

- [ ] steering_cmd outport dimensions: [5x1]

- [ ] IPGMovie_Capture wired to LKA_System camera_img in generic.mdl

## Algorithm Parameters

- [ ] xm_per_pix = 0.0083 (matches BEV calibration for 2m lane)

- [ ] BEV src points match actual camera view

- [ ] alpha = 0.20 (temporal smoothing)

- [ ] k_curv = 800 (pixel curvature units)

- [ ] Width sanity check: 240 < w < 600 px